ALU 算术逻辑单元设计

1. 实验目的

- 掌握 ALU 的基本功能、工作原理和硬件描述方法。

- 掌握算术运算、逻辑运算、移位运算与状态标志位之间的关系。

- 熟悉 Vivado 中 VHDL 工程建立、行为仿真、波形观察和结果验证流程。

- 实验基本原理及内容

- ALU 功能与结构：

算术逻辑单元（ALU）:执行各种算术和逻辑运算。

算术运算操作 :加、减、乘、除

逻辑运算操作:与、或、非、异或

ALU 输入:操作数以及来自控制单元的控制命令

ALU 输出:运算结果，以及状态信息

- 基于器件的 8 位 ALU 设计

采用硬件描述语言语言设计 74373 及 74181 模块，并通过相应的接口将两个模块连接在一

起形成 8 位 ALU 模块。如下图所示：

- 采用硬件描述语言语言设计 ALU 模块

具体端口定义如下：

entity alu is

Port (m_a,m_b:in std_logic_vector(7 downto 0);

c0:in std_logic;

s:in std_logic_vector(4 downto 0);

f_in:in std_logic_vector(1 downto 0);

data: out std_logic_vector(7 downto 0);

cy,zn: out std_logic );

end alu;

（4）设计原理与模块划分

ALU 端口

| 端口 | 方向 | 位宽 | 说明 |
| --- | --- | --- | --- |
| m_a | in | 8 | 操作数 A |
| m_b | in | 8 | 操作数 B |
| c0 | in | 1 | 进位/借位控制输入 |
| s | in | 5 | 运算选择，s[4] 为模式位 |
| f_in | in | 2 | 输出移位/循环移位选择 |
| data | out | 8 | 运算结果 |
| cy | out | 1 | 进位标志 |
| zn | out | 1 | 零标志 |

运算控制表

| S[3:0] | M=0 算术功能 | M=1 逻辑功能 |
| --- | --- | --- |
| 0000 | A + C0 | NOT A |
| 0001 | (A OR B) + C0 | NOT(A OR B) |
| 0010 | (A OR NOT B) + C0 | (NOT A) AND B |
| 0011 | 0 - C0 | 0 |
| 0100 | A + (A AND NOT B) + C0 | NOT(A AND B) |
| 0101 | (A OR B) + (A AND NOT B) + C0 | NOT B |
| 0110 | A - B - C0 | A XOR B |
| 0111 | (A OR NOT B) - C0 | A AND NOT B |
| 1000 | A + (A AND B) + C0 | (NOT A) AND B |
| 1001 | A + B + C0 | NOT(A XOR B) |
| 1010 | (A OR NOT B) + (A AND B) + C0 | B |
| 1011 | (A AND B) - C0 | A AND B |
| 1100 | A + A + C0 | 1 |
| 1101 | (A OR B) + A + C0 | A OR NOT B |
| 1110 | (A OR NOT B) + A + C0 | A OR B |
| 1111 | A - C0 | A |

输出处理控制

| f_in | 输出处理 | VHDL 表达式 |
| --- | --- | --- |
| 00 | 直通输出 | result <= f |
| 01 | 循环右移一位 | result <= f(0) & f(7 downto 1) |
| 10 | 循环左移一位 | result <= f(6 downto 0) & f(7) |
| 11 | 逻辑左移一位 | result <= f(6 downto 0) & '0' |

3. 实验源码

design_sources\cal.vhd

LIBRARY IEEE;

USE IEEE.STD_LOGIC_1164.ALL;

USE IEEE.STD_LOGIC_UNSIGNED.ALL;

ENTITY cal IS

PORT (

S  : IN  STD_LOGIC_VECTOR(3 DOWNTO 0 );

A  : IN  STD_LOGIC_VECTOR(7 DOWNTO 0);

B  : IN  STD_LOGIC_VECTOR(7 DOWNTO 0);

F  : OUT STD_LOGIC_VECTOR(7 DOWNTO 0);

M  : IN  STD_LOGIC;

CN : IN  STD_LOGIC;

CO : OUT STD_LOGIC   );

END cal;

ARCHITECTURE behav OF cal IS

SIGNAL A9 : STD_LOGIC_VECTOR(8 DOWNTO 0);

SIGNAL B9 : STD_LOGIC_VECTOR(8 DOWNTO 0);

SIGNAL F9 : STD_LOGIC_VECTOR(8 DOWNTO 0);

BEGIN

A9 <= '0' & A ;  B9 <= '0' & B ;

PROCESS(S,M,CN,A9,B9)

BEGIN

CASE S  IS

WHEN "0000" =>  IF M='0' THEN F9<=A9 + CN                        ; ELSE  F9<=NOT A9;                END IF;

WHEN "0001" =>  IF M='0' THEN F9<=(A9 or B9) + CN                ; ELSE  F9<=NOT(A9 OR B9);         END IF;

WHEN "0010" =>  IF M='0' THEN F9<=(A9 or (NOT B9))+ CN           ; ELSE  F9<=(NOT A9) AND B9;       END IF;

WHEN "0011" =>  IF M='0' THEN F9<= "000000000" - CN              ; ELSE  F9<="000000000";           END IF;

WHEN "0100" =>  IF M='0' THEN F9<=A9+(A9 AND NOT B9)+ CN         ; ELSE  F9<=NOT (A9 AND B9);       END IF;

WHEN "0101" =>  IF M='0' THEN F9<=(A9 or B9)+(A9 AND NOT B9)+CN  ; ELSE  F9<=NOT B9;                END IF;

WHEN "0110" =>  IF M='0' THEN F9<=(A9 - B9) - CN                 ; ELSE  F9<=A9 XOR B9;             END IF;

WHEN "0111" =>  IF M='0' THEN F9<=(A9 or (NOT B9)) - CN          ; ELSE  F9<=A9 and (NOT B9);       END IF;

WHEN "1000" =>  IF M='0' THEN F9<=A9 + (A9 AND B9)+CN            ; ELSE  F9<=(NOT A9)and B9;        END IF;

WHEN "1001" =>  IF M='0' THEN F9<=A9 + B9 + CN                   ; ELSE  F9<=NOT(A9 XOR B9);        END IF;

WHEN "1010" =>  IF M='0' THEN F9<=(A9 or(NOT B9))+(A9 AND B9)+CN ; ELSE  F9<=B9;                    END IF;

WHEN "1011" =>  IF M='0' THEN F9<=(A9 AND B9)- CN                ; ELSE  F9<=A9 AND B9;             END IF;

WHEN "1100" =>  IF M='0' THEN F9<=(A9 + A9) + CN                 ; ELSE  F9<= "000000001";          END IF;

WHEN "1101" =>  IF M='0' THEN F9<=(A9 or B9) + A9 + CN           ; ELSE  F9<=A9 OR (NOT B9);        END IF;

WHEN "1110" =>  IF M='0' THEN F9<=((A9 or (NOT B9)) +A9) + CN    ; ELSE  F9<=A9 OR B9;              END IF;

WHEN "1111" =>  IF M='0' THEN F9<=A9 - CN                        ; ELSE  F9<=A9 ;                   END IF;

WHEN OTHERS  => F9<= "000000000" ;

END CASE;

END PROCESS;

F<= F9(7 DOWNTO 0) ;   CO <= F9(8) ;

END behav;

design_sources\alu.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

-- Uncomment the following library declaration if using

-- arithmetic functions with Signed or Unsigned values

--use IEEE.NUMERIC_STD.ALL;

-- Uncomment the following library declaration if instantiating

-- any Xilinx leaf cells in this code.

--library UNISIM;

--use UNISIM.VComponents.all;

entity alu is

Port (m_a,m_b:in std_logic_vector(7 downto 0);

c0:in std_logic;

s:in std_logic_vector(4 downto 0);

f_in:in std_logic_vector(1 downto 0);

data: out std_logic_vector(7 downto 0);

cy,zn: out std_logic );

end alu;

architecture Behavioral of alu is

signal a,b,result,f:std_logic_vector(7 downto 0);

signal co,acc,cyy,znn,ovv:std_logic;

component cal is

PORT (

S  : IN  STD_LOGIC_VECTOR(3 DOWNTO 0 );

A  : IN  STD_LOGIC_VECTOR(7 DOWNTO 0);

B  : IN  STD_LOGIC_VECTOR(7 DOWNTO 0);

F  : OUT STD_LOGIC_VECTOR(7 DOWNTO 0);

M  : IN  STD_LOGIC;

CN : IN  STD_LOGIC;

CO : OUT STD_LOGIC   );

END component;

begin

ls181: cal port map(S=>s(3 downto 0),A=>a,B=>b,F=>f,M=>s(4),CN=>c0,CO=>co);

a<=m_a;b<=m_b;data<=result;

process(f, f_in)

begin

case f_in is

when "00"=>result<=f;

when "01"=>result<=f(0) & f(7 downto 1);

when "10"=>result<=f(6 downto 0) & f(7);

when "11"=>result<=f(6 downto 0) & '0';

when others=>result<=f;

end case;

end process;

process(co,f)

begin

if f="00000000" then zn<='1'; else zn<='0'; end if;

cy<=co;

end process;

end Behavioral;

design_sources\data2seg.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

-- Uncomment the following library declaration if using

-- arithmetic functions with Signed or Unsigned values

--use IEEE.NUMERIC_STD.ALL;

-- Uncomment the following library declaration if instantiating

-- any Xilinx leaf cells in this code.

--library UNISIM;

--use UNISIM.VComponents.all;

entity data2seg is

Port (data_in:in std_logic_vector(3 downto 0);

seg_data:out std_logic_vector(7 downto 0));

end data2seg;

architecture Behavioral of data2seg is

begin

process(data_in)

begin

case data_in is

when x"0"=>seg_data<=x"c0";

when x"1"=>seg_data<=x"f9";

when x"2"=>seg_data<=x"a4";

when x"3"=>seg_data<=x"b0";

when x"4"=>seg_data<=x"99";

when x"5"=>seg_data<=x"92";

when x"6"=>seg_data<=x"82";

when x"7"=>seg_data<=x"f8";

when x"8"=>seg_data<=x"80";

when x"9"=>seg_data<=x"90";

when x"a"=>seg_data<=x"88";

when x"b"=>seg_data<=x"83";

when x"c"=>seg_data<=x"c6";

when x"d"=>seg_data<=x"a1";

when x"e"=>seg_data<=x"86";

when x"f"=>seg_data<=x"8e";

when others=>seg_data<=x"00";

end case;

end process;

end Behavioral;

design_sources\state.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_unsigned.all;

-- Uncomment the following library declaration if using

-- arithmetic functions with Signed or Unsigned values

--use IEEE.NUMERIC_STD.ALL;

-- Uncomment the following library declaration if instantiating

-- any Xilinx leaf cells in this code.

--library UNISIM;

--use UNISIM.VComponents.all;

entity state is

Port (clk, rst : in std_logic;

seg_sel : out std_logic_vector(15 downto 0);

seg_data : out std_logic_vector(7 downto 0);

show:in std_logic_vector(63 downto 0) );

end state;

architecture Behavioral of state is

type states is (s0,s1,s2,s3,s4,s5,s6,s7,s8,s9,s10,s11,s12,s13,s14,s15);

signal state,next_state:states:=s0;

signal show1: std_logic_vector(3 downto 0);

signal clks,cc: std_logic:='0';

signal showw:std_logic_vector(63 downto 0):=(others=>'0');

component data2seg is

Port (data_in:in std_logic_vector(3 downto 0);

seg_data:out std_logic_vector(7 downto 0));

end component;

begin

u: data2seg port map(data_in=>show1, seg_data=>seg_data);

process(clk,rst)

variable count: integer range 0 to 30000:=0;

begin

if rst='1' then

count:=0;

clks<='0';

elsif clk'event and clk='1' then

count:=count+1;

if count=10000 then

clks<=not clks;

count:=0;

end if;

end if;

end process;

process(clks,rst)    --复位和状态转移

begin

if rst='1' then

state<=s0;showw<=x"0123456789abcdef";

elsif clks'event and clks='1' then

showw<=show;

state<=next_state;

end if;

end process;

process(state,showw)

begin

case state is --数码管输出

when s0=>

seg_sel<=x"fffe"; show1<=showw(3 downto 0); next_state<=s1;

when s1=>

seg_sel<=x"fffd"; show1<=showw(7 downto 4); next_state<=s2;

when s2=>

seg_sel<=x"fffb"; show1<=showw(11 downto 8); next_state<=s3;

when s3=>

seg_sel<=x"fff7"; show1<=showw(15 downto 12); next_state<=s4;

when s4=>

seg_sel<=x"ffef"; show1<=showw(19 downto 16);  next_state<=s5;

when s5=>

seg_sel<=x"ffdf"; show1<=showw(23 downto 20); next_state<=s6;

when s6=>

seg_sel<=x"ffbf"; show1<=showw(27 downto 24);next_state<=s7;

when s7=>

seg_sel<=x"ff7f"; show1<=showw(31 downto 28); next_state<=s8;

when s8=>

seg_sel<=x"feff"; show1<=showw(35 downto 32); next_state<=s9;

when s9=>

seg_sel<=x"fdff"; show1<=showw(39 downto 36); next_state<=s10;

when s10=>

seg_sel<=x"fbff"; show1<=showw(43 downto 40);  next_state<=s11;

when s11=>

seg_sel<=x"f7ff"; show1<=showw(47 downto 44); next_state<=s12;

when s12=>

seg_sel<=x"efff"; show1<=showw(51 downto 48); next_state<=s13;

when s13=>

seg_sel<=x"dfff"; show1<=showw(55 downto 52); next_state<=s14;

when s14=>

seg_sel<=x"bfff"; show1<=showw(59 downto 56); next_state<=s15;

when s15=>

seg_sel<=x"7fff"; show1<=showw(63 downto 60); next_state<=s0;

end case;

end process;

end Behavioral;

design_sources\test.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

-- Uncomment the following library declaration if using

-- arithmetic functions with Signed or Unsigned values

--use IEEE.NUMERIC_STD.ALL;

-- Uncomment the following library declaration if instantiating

-- any Xilinx leaf cells in this code.

--library UNISIM;

--use UNISIM.VComponents.all;

entity test is

Port (clk, rst : in std_logic;

seg_sel : out std_logic_vector(15 downto 0);

seg_data : out std_logic_vector(7 downto 0);

input1,input2:in std_logic_vector(7 downto 0);

s:in std_logic_vector(4 downto 0);

f_in:in std_logic_vector(1 downto 0);

c0:in std_logic;

cy,zn:out std_logic

);

end test;

architecture Behavioral of test is

component alu is

Port (m_a,m_b:in std_logic_vector(7 downto 0);

c0:in std_logic;

s:in std_logic_vector(4 downto 0);

f_in:in std_logic_vector(1 downto 0);

data: out std_logic_vector(7 downto 0);

cy,zn: out std_logic );

end component;

component state is

Port (clk, rst : in std_logic;

seg_sel : out std_logic_vector(15 downto 0);

seg_data : out std_logic_vector(7 downto 0);

show:in std_logic_vector(63 downto 0) );

end component;

signal showw: std_logic_vector(63 downto 0);

begin

showw(63 downto 24)<=(others=>'0');

showw(15 downto 8)<=input2;

showw(23 downto 16)<=input1;

u1:alu port map(m_a=>input1,m_b=>input2,c0=>c0,s=>s,f_in=>f_in,data=>showw(7 downto 0),cy=>cy,zn=>zn);

u2:state port map(clk=>clk,rst=>rst,seg_sel=>seg_sel,seg_data=>seg_data,show=>showw);

end Behavioral;

simulation_sources\alu_tb.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity alu_tb is

end alu_tb;

architecture sim of alu_tb is

component alu is

Port (

m_a  : in  std_logic_vector(7 downto 0);

m_b  : in  std_logic_vector(7 downto 0);

c0   : in  std_logic;

s    : in  std_logic_vector(4 downto 0);

f_in : in  std_logic_vector(1 downto 0);

data : out std_logic_vector(7 downto 0);

cy   : out std_logic;

zn   : out std_logic

);

end component;

signal m_a_tb  : std_logic_vector(7 downto 0) := (others => '0');

signal m_b_tb  : std_logic_vector(7 downto 0) := (others => '0');

signal c0_tb   : std_logic := '0';

signal s_tb    : std_logic_vector(4 downto 0) := (others => '0');

signal f_in_tb : std_logic_vector(1 downto 0) := "00";

signal data_tb : std_logic_vector(7 downto 0);

signal cy_tb   : std_logic;

signal zn_tb   : std_logic;

begin

uut: alu

port map (

m_a  => m_a_tb,

m_b  => m_b_tb,

c0   => c0_tb,

s    => s_tb,

f_in => f_in_tb,

data => data_tb,

cy   => cy_tb,

zn   => zn_tb

);

stimulus: process

begin

-- Arithmetic: A + B + C0. s(4)=0, s(3 downto 0)=1001.

m_a_tb <= x"03"; m_b_tb <= x"03"; s_tb <= "01001"; c0_tb <= '0'; f_in_tb <= "00";

wait for 20 ns;

assert data_tb = x"06" and cy_tb = '0' and zn_tb = '0'

report "A+B failed" severity error;

c0_tb <= '1';

wait for 20 ns;

assert data_tb = x"07" and cy_tb = '0' and zn_tb = '0'

report "A+B+C0 failed" severity error;

-- Logic: NOT(A XOR B). s(4)=1, s(3 downto 0)=1001.

s_tb <= "11001"; c0_tb <= '0';

wait for 20 ns;

assert data_tb = x"FF" and cy_tb = '1' and zn_tb = '0'

report "XNOR failed" severity error;

-- Arithmetic: A - B - C0. s(4)=0, s(3 downto 0)=0110.

m_a_tb <= x"B3"; m_b_tb <= x"A1"; s_tb <= "00110"; c0_tb <= '0';

wait for 20 ns;

assert data_tb = x"12" and cy_tb = '0' and zn_tb = '0'

report "A-B failed" severity error;

c0_tb <= '1';

wait for 20 ns;

assert data_tb = x"11" and cy_tb = '0' and zn_tb = '0'

report "A-B-C0 failed" severity error;

-- Logic: A XOR B. s(4)=1, s(3 downto 0)=0110.

s_tb <= "10110"; c0_tb <= '0';

wait for 20 ns;

assert data_tb = x"12" and cy_tb = '0' and zn_tb = '0'

report "XOR failed" severity error;

-- Shift and rotate stage after the ALU output.

m_a_tb <= x"81"; m_b_tb <= x"01"; s_tb <= "01001"; c0_tb <= '0'; f_in_tb <= "01";

wait for 20 ns;

assert data_tb = x"41"

report "rotate right failed" severity error;

f_in_tb <= "10";

wait for 20 ns;

assert data_tb = x"05"

report "rotate left failed" severity error;

f_in_tb <= "11";

wait for 20 ns;

assert data_tb = x"04"

report "logical left shift failed" severity error;

-- Zero flag.

m_a_tb <= x"00"; m_b_tb <= x"00"; s_tb <= "01001"; c0_tb <= '0'; f_in_tb <= "00";

wait for 20 ns;

assert data_tb = x"00" and zn_tb = '1'

report "zero flag failed" severity error;

assert false report "ALU simulation finished" severity note;

wait;

end process;

end sim;

constraints\pin.xdc

# Clock

create_clock -period 10.000 -name sys_clk [get_ports clk]

# input1 (SW0~SW7)

set_property PACKAGE_PIN C9  [get_ports {input1[0]}]

set_property PACKAGE_PIN B9  [get_ports {input1[1]}]

set_property PACKAGE_PIN G11 [get_ports {input1[2]}]

set_property PACKAGE_PIN F10 [get_ports {input1[3]}]

set_property PACKAGE_PIN D10 [get_ports {input1[4]}]

set_property PACKAGE_PIN E11 [get_ports {input1[5]}]

set_property PACKAGE_PIN D11 [get_ports {input1[6]}]

set_property PACKAGE_PIN A14 [get_ports {input1[7]}]

set_property IOSTANDARD LVCMOS18 [get_ports {input1*}]

# input2 (SW8~SW15)

set_property PACKAGE_PIN B10 [get_ports {input2[0]}]

set_property PACKAGE_PIN A10 [get_ports {input2[1]}]

set_property PACKAGE_PIN B15 [get_ports {input2[2]}]

set_property PACKAGE_PIN A15 [get_ports {input2[3]}]

set_property PACKAGE_PIN A13 [get_ports {input2[4]}]

set_property PACKAGE_PIN A12 [get_ports {input2[5]}]

set_property PACKAGE_PIN D8  [get_ports {input2[6]}]

set_property PACKAGE_PIN D9  [get_ports {input2[7]}]

set_property IOSTANDARD LVCMOS18 [get_ports {input2*}]

# s[4:0] (SW16~SW20)

set_property PACKAGE_PIN F8  [get_ports {s[0]}]

set_property PACKAGE_PIN F9  [get_ports {s[1]}]

set_property PACKAGE_PIN H11 [get_ports {s[2]}]

set_property PACKAGE_PIN H12 [get_ports {s[3]}]

set_property PACKAGE_PIN G14 [get_ports {s[4]}]

set_property IOSTANDARD LVCMOS18 [get_ports {s*}]

# f_in (SW21~SW22)

set_property PACKAGE_PIN J10 [get_ports {f_in[0]}]

set_property PACKAGE_PIN H14 [get_ports {f_in[1]}]

set_property IOSTANDARD LVCMOS18 [get_ports {f_in*}]

# c0 (SW23)

set_property PACKAGE_PIN J11 [get_ports c0]

set_property IOSTANDARD LVCMOS18 [get_ports c0]

# rst (SW31)

set_property PACKAGE_PIN J8 [get_ports rst]

set_property IOSTANDARD LVCMOS18 [get_ports rst]

# clk

set_property PACKAGE_PIN E10 [get_ports clk]

set_property IOSTANDARD LVCMOS18 [get_ports clk]

# cy, zn (LD0, LD1)

set_property PACKAGE_PIN G16 [get_ports cy]

set_property PACKAGE_PIN H16 [get_ports zn]

set_property IOSTANDARD LVCMOS18 [get_ports cy]

set_property IOSTANDARD LVCMOS18 [get_ports zn]

# seg_data[7:0]

set_property PACKAGE_PIN J21 [get_ports {seg_data[0]}]

set_property PACKAGE_PIN H24 [get_ports {seg_data[1]}]

set_property PACKAGE_PIN H23 [get_ports {seg_data[2]}]

set_property PACKAGE_PIN G21 [get_ports {seg_data[3]}]

set_property PACKAGE_PIN H21 [get_ports {seg_data[4]}]

set_property PACKAGE_PIN H26 [get_ports {seg_data[5]}]

set_property PACKAGE_PIN J26 [get_ports {seg_data[6]}]

set_property PACKAGE_PIN E26 [get_ports {seg_data[7]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg_data*}]

# seg_sel[15:0]

set_property PACKAGE_PIN B20 [get_ports {seg_sel[0]}]

set_property PACKAGE_PIN C22 [get_ports {seg_sel[1]}]

set_property PACKAGE_PIN D21 [get_ports {seg_sel[2]}]

set_property PACKAGE_PIN C24 [get_ports {seg_sel[3]}]

set_property PACKAGE_PIN B26 [get_ports {seg_sel[4]}]

set_property PACKAGE_PIN B25 [get_ports {seg_sel[5]}]

set_property PACKAGE_PIN L23 [get_ports {seg_sel[6]}]

set_property PACKAGE_PIN K21 [get_ports {seg_sel[7]}]

set_property PACKAGE_PIN H22 [get_ports {seg_sel[8]}]

set_property PACKAGE_PIN J24 [get_ports {seg_sel[9]}]

set_property PACKAGE_PIN J25 [get_ports {seg_sel[10]}]

set_property PACKAGE_PIN A20 [get_ports {seg_sel[11]}]

set_property PACKAGE_PIN C26 [get_ports {seg_sel[12]}]

set_property PACKAGE_PIN D26 [get_ports {seg_sel[13]}]

set_property PACKAGE_PIN A24 [get_ports {seg_sel[14]}]

set_property PACKAGE_PIN A23 [get_ports {seg_sel[15]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg_sel*}]

- 4.仿真结果与分析

仿真波形中，data_tb 依次输出 06、07、FF、12、11、12、41、05、04、00，与 testbench 断言的期望值一致，说明加法、带进位加法、同或、减法、带借位减法、异或、循环右移、循环左移、逻辑左移和零标志均通过验证。

5. 实验结论

本实验完成了 8 位 ALU 的 VHDL 设计与仿真验证。设计能够在 5 位控制信号作用下完成多种算术和逻辑运算，并支持结果直通、循环右移、循环左移和逻辑左移处理。仿真结果与理论计算一致，满足 ALU 算术逻辑单元设计要求。